Quarch Technology Ltd

AN-012

Application Note

QIS Python control of power modules

For use with:

Original Power Modules
XLC Power Modules
HD Power Modules

# Change History

| -01 | Dec 2016 | Initial Release |
| --- | --- | --- |
| -02 | Feb 2017 | Improved documentation |
| -03 | March 2017 | Fixed bug where LAN scan port could be set incorrectly |
| -04 | November 2017 | Improved stability and simplified user interface. Addition of stream averaging Addition of multiport control with single script |

Contents

Change History	2

Introduction	4

Modules Supported	6

Installation and setup	7

Linux setup	7

Windows setup	10

Provided Examples	12

Locating devices	12

Streaming capture example	13

QIS built in terminal	14

QIS remote terminal control	18

Customer support from Quarch	20

Access support from the Quarch website	20

Find discussion topics, support information and testing ideas.	20

# Introduction

Quarch Programmable Power Modules (PPMs) can be controlled using QIS (the Quarch Instrumentation Server). This is a simple Java server application which runs in the background and allows simple control of Quarch modules.

While these modules can be controlled via pure python (AN-004 application note), there are advantages to using QIS:

No additional library installs are required (other than Java)

Power modules can stream at far higher rates with QIS than with pure python

QIS hides much of the complexity of binary data transfer from power modules

QIS can be controlled from any language (via TCP port), so use is not restricted to Python

QIS can use multiple devices at the same time, across different interfaces

QIS can locate remote LAN devices without knowing their IP or name (using a TCP broadcast packet)

QIS runs on any Windows/Linux host PC. A USB cable or network connection to the Quarch power module allows QIS to control it.

The automated scripts that control QIS can now be run on the Host PC, or on any other PC on the network. While this application note demonstrates the use of Python, any language can be used to connect to QIS over TCP and issue commands.

QIS

Customer Drive

USB/LAN Connection

Power Module

Customer Host PC

# Modules Supported

Original power modules:

QTL1455

QTL1658

QTL1730

QTL1727

XLC power modules

QTL1824 (QTL1824-02A modules do NOT support streaming, but can be upgraded by Quarch)

QTL1847

HD power modules

QTL1995

QTL1999

All standard Quarch modules and controllers are also supported by QIS, though this note is specifically looking at control of power modules.

# Installation and setup

## Linux setup

This setup was based on Ubuntu Linux and a clean install. Changes may be needed for other systems. You may require admin access for the install.

Copy the application note files to a folder on your Linux system

Check Java is installed:

- Open a terminal window

- >java -version

If the command fails, you will need to install the appropriate version of the Java JRE on your system.

You can find install instructions and files here:
http://www.oracle.com/technetwork/java/javase/downloads/index.html

Run QIS

QIS is found in the /QIS folder of the application note. Simple double click QIS.jar to run it

QIS can also be invoked from the command line:

>java –jar qis.jar

For systems without a GUI environment, you can disable all graphical components, allowing the server to run on basic systems.

>java –Djava.awt.headless=true –jar qis.jar

If running QIS from a terminal window or script, you will likely find that closing the window will terminate QIS, you can prevent this from happening by using a combination of ‘nohup’ and ‘&’ on the command:

>nohup java –jar qis.jar &

Unless you have used the ‘headless’ option, you should now see a small Quarch icon in the task bar, representing QIS. Right clicking on this gives options to exit or to display a manual control terminal for typing commands.

Default USB permissions on Ubuntu may require ‘sudo’ to run QIS, or it will not have access to USB. This can be avoided by setting explicit permissions for Quarch USB devices.

Create a text file called Quarch-Permissions-USB.rules

Into the file, enter the text:

SUBSYSTEM==”usb”,ATTRS{idVendor}==”16d0”,MODE=”0666”
	SUBSYSTEM==”usb_device”,ATTRS{idVendor}==”16d0”,MODE=”0666”

Save the file and close it

On the command line, copy it into the rules folder:

>sudo cp Quarch-Permissions-USB.rules /etc/udev/rules.d

You will need to reset for this to take effect

Run the list devices script, to check which modules are available to you

>python QisListDevices.py

Below we can see that three devices were found, one local XLC module on USB and two remote devices on LAN.

If you have not run QIS before running the script, you instead will see:

You are now fully ready to run the main examples

## Windows setup

This setup was based on Windows 10 and a clean install. Changes may be needed for other systems. You may require admin access for the install.

  - Copy the application note files to a folder on your system

Download and install the Quarch USB driver from:

- http://quarch.com/downloads?field_file_type_tid=113

Check that the Java JRE is installed

- You can find install instructions and files here:
http://www.oracle.com/technetwork/java/javase/downloads/index.html

Run QIS

QIS is found in the /QIS folder of the application note. Simple double click QIS.jar to run it

QIS can also be invoked from the command line:

>java –jar qis.jar

For systems without a GUI environment, you can disable all graphical components, allowing the server to run on basic systems.

>java –Djava.awt.headless=true –jar qis.jar

Unless you have used the ‘headless’ option, you should now see a small Quarch icon in the task bar, representing QIS. Right clicking on this gives options to exit or to display a manual control terminal for typing commands.

Run the list devices script, to check which modules are available to you

>python QisListDevices.py

Below we can see that three devices were found, one local XLC module on USB and two remote devices on LAN.

If you have not run QIS before running the script, you instead will see:

You are now fully ready to run the main examples

# Provided Examples

## Locating devices

The file QisListDevices.py contains a simple example, querying QIS to get a list of available devices. This lets us see what is available, and conform that everything is working.

The name format will be used later for choosing the module to use. For example:

usb::qtl1847-01-016

This represents a USB connection to a QTL1847 (XLC Power Module), with a serial number of QTL1847-01-016

>python QisListDevices.py

## Streaming capture example

The file QisStreamExample.py connects to a selected module and runs a series of commands. These are split up into a few separate examples that are designed to do slightly different things.

The “powerMarginingExample” function demonstrates how the voltage being produced by the power module can be altered and measured without any streaming being involved. As no streaming is being performed, it requires much less CPU usage with the downside of simplistic power measurement.

The “simpleStream” function demonstrates how data can be streamed from the module. ‘Streaming’ involves the PC client software constantly puling data back from the Quarch module. If the PC client did not pull data from the module, its internal buffer would fill up and streaming would halt.

The module samples the voltage and current every 4µs and has the capability of averaging those samples before sending the data to the PC. An XLC, on a fast PC, can generally keep up with 16 samples being averaged in the device, whereas an HD can handle 4 samples. If any lower device averaging rates are selected, the device cannot transmit data fast enough and so its RAM will fill up. “simpleStream” uses the device default for averaging, which is no averaging or zero averaging. To change the device averaging rate, the command:

qis.sendCmd(module, "Record Averaging x")

Where x is any whole 2n from 0 to 32k.

“multiStreamExample” demonstrates a simple method for handling buffer overrun. This occurs when the device averaging bandwidth is greater than the rate data can be saved to file. When any overrun is detected, in either the module or QIS, the stream is stopped and restarted, saving the data into a new file.

Note: It can take time to empty either the QIS or device buffers so there will be sections of time where no data is present so if continuous data is required, use a high enough device sample rate such that overrun does not occur.

The “averageStream” function demonstrates the script averaging feature built into QisInterface.py. This feature takes samples from the device and further averages them to create a user defined time base. In the example, data points will be saved to file that represent a 1s worth of data. All that needs to be set by the user is the final required averaging rate, and the averaging rate performed by the device.

Note: The implementation used has an absolute timing error for each data point equal to: Percentage error= Device averaging rate ×4μsScript averaging rate For example, having each data point represent 1s with a device averaging rate of 1k has a timing error of ±0.4096. This timing error does not translate to a cumulative timing error so 100 data points will represent 100s of time, in this case.

Finally, the multiDeviceStreamExample function demonstrates how multiple modules can be controlled with a single script.

Note: As more devices stream to the same PC, the maximum achievable data rate is reduced. This means, buffer overrun will occur at higher device averaging rates.

The example function demonstrates how all 6 ports of a QTL1999 6 port HD module can be communicated with and stream from each port simultaneously.

These examples all use a number of functions built into this script or QisInterface.py. These functions are:

debugPrint(string, setting)

Prints string to console or file dependant on debugPrintSetup. When setting = 1 the string can be saved to file but will not print to console.

debugPrintSetup(setting, filename)

Setting can be either “Command Line” or “File”. Filename is required when saving to file.

qis.sendCmd(module, command)

Sends commands to the module via QIS. Module is the net bios or IP address of the module being communicated with. Command is the string being sent to the module. It returns text from the device dependent on the command.

qis.startStream(module, filename, fileMaxMB, streamName, streamAverage)

Starts device streaming. module is the module being commanded. fileMaxMB is the max file size for streamed data files. streamName is saved into the header for the stream data file. streamAverage is the script average performed in seconds. Can be set to None if not required.

qis.streamRunningStatus(module)

Returns the running status of the stream from the module

# Debugging

There are a number of built in options for debugging QIS and Python scripts, allowing you to track down problems

The simple script to ‘locate devices’ (described above) checks that QIS is running and accessible, without needing a device attached

QIS has a built in terminal which can be used to check access to devices, without needing to run a script

A remote terminal (putty for example) can be used to check access to a QIS server, allowing you to check it is accessible needing access to Python

## QIS built in terminal

QIS has a built in Terminal. When running, there will be an icon in the task bar. Right click this and select ‘Show Terminal’. If the icon does not work on your linux distro, run QIS with the –terminal option

To get a list of commands enter:

>$help

To get a list of available modules enter:

>$list

To select a device to control (We want the USB device in this case), use the command:

>$default 1

You can now enter any standard command for the chosen device (see the device technical manual for command lists).

## QIS remote terminal control

QIS works by exposing a TCP port on the PC it runs on. You can connect to this via a standard terminal program such as Putty. Below you can see the setting used.

The IP address is the IP of the PC running QIS
The port is 9722
The connection type must be ‘Raw’

When connected, you can use the same commands as were described in the ‘QIS built in terminal’ section.

## Know Issues

CentOS 7 with Gnome:

- Running QIS will show an icon for the server application, but it cannot be right-clicked to get to the menu.

- To use the terminal, it must be invoked with: 
java –jar qis.jar –terminal

When the terminal is running, the icon on the task bar is over sized, but is functional. If the terminal is closed, you will not be able to open it again.

As the taskbar icon does not work, you may have to terminate the QIS process manually.

# Customer support from Quarch

There are multiple ways to access the support you need. You can contact us directly or access an extensive range of valuable support materials from http://quarch.com/support.

Contact us direct

Get going quickly and easily, with help direct from the engineers:

Call +44 1343 508 140 or email support@quarch.com during UK office hours.

Our international partners are well trained in the use of our products and can deal with many basic technical queries from within your time zone, if you prefer. Check http://quarch.com/resellers for the contact details of your regional supplier.

## Access support from the Quarch website

You can download up-to-date software and drivers, technical manuals, datasheets and more from our website. To help you get started quickly we provide additional documents, such as examples in Perl, Python and C# and Telnet and Serial instructions.

Key places to visit on the Quarch website

Register your Quarch product to confirm your international warranty: http://quarch.com/product-registration

Download a wide range of documentation, free applications and drivers to help you make the best possible use of your Quarch tools: http://quarch.com/content/downloads

Access the Quarch support forum (http://quarch.com/forum):

## Find discussion topics, support information and testing ideas.

Browse existing topics or login to your user account to ask for information and advice.

Sign up for Quarch Technical Updates to get the most out of your Quarch products. Updates are published approximately once a quarter and include news about the latest features, tools, application notes and software updates. See http://quarch.com/content/sign-quarch-technical-updates.